# ANNEX II + III:	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

# Contract title: Supply of specialized equipment for the needs of the Law enforcement departments in Edirne	p 1 /10

Publication reference: BGTR0100002-SUPPLY1

Columns 1-2 should be completed by the contracting authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the contracting authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words ‘compliant’ or ‘yes’ are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

| 1. Item number | 2. Specifications required | 3. Specifications offered | 4. Notes, remarks, ref to documentation | 5. Evaluation committee’s notes |
| --- | --- | --- | --- | --- |
| 1 | Forged Document Inspection Device (Basic) Light sources: white oblique - white bottom - ultraviolet 365 nm Detection of security features with magnetic properties - Maximum document size, mm 210×300 (А4) - Dimensions (length×width×height), mm 280×190×180 Weight, kg 1,9 Power supply, V 12 Power consumption, W, max 18 |  |  |  |
| 2 | Forged Document Inspection Device (Advanced) Magnification: up to 30x on-screen Frame size: 1024×638 (standart mode), 2048×1276 (High resolution mode) Video camera — 5 Mp, CMOS Frame size, pixels: standard mode — 1024×638 HDR mode — 2048×1276 Optical magnification, times — up to 25 Magnification on the monitor, times — up to 30 Field of view: maximum, mm — 180×112 minimum, mm — 7×4.3 |  |  |  |
| 3 | Passport Optic Reader Overall dimensions (length×width×height), mm: 179×160×99 Weight, not more than, kg 0,82 Power supply voltage from a USB port, V 5 Power supply voltage from AC adapter (AC 100-240 V / DC 5 V) optionally Scratch resistant glass (Sapphire) Regulatory CE — RED, LVD & EMC EU WEEE, Climatic conditions Relative air humidity 20...95% Air temperature,˚С -10...+50 IP51 |  |  |  |
| 4 | Camera (Advanced) Image Sensor 1/1.8"" Progressive Scan CMOS Max. Resolution 1920 × 1080 Min. Illumination Color: 0.0005 Lux @ (F1.2, AGC ON); B/W: 0.0001 Lux @ (F1.2, AGC ON)，0 Lux with IR Shutter Time 1 s to 1/100,000 s Day & Night IR cut filter Lens Focal Length & FOV 2.8 to 12 mm, horizontal FOV: 114.5° to 41.8°, vertical FOV: 59.3° to 23.6°, diagonal FOV: 141.1° to 48° 8 to 32 mm, horizontal FOV: 42.5° to 15.1°, vertical FOV: 23.3° to 8.64°, diagonal FOV: 49.6° to 17.3° Focus Auto, semi-auto, manual Iris Type P-iris Aperture 2.8 to 12 mm: F1.2 to F2.5 8 to 32 mm: F1.7 to F1.73" |  |  |  |
| 5 | Camera (Basic) Image Sensor 1/2.8"" 2Megapixel progressive scan CMOS Effective Pixels 1920 (H) x1080 (V) RAM / ROM 1024MB / 128MB Scanning System Progressive Electronic Shutter Speed Auto / Manual, 1/3 ~ 1 / 100000s Minimum Illumination 0.005Lux / F1.4 (Color, 1 / 3s, 30IRE) 0.03Lux / F1.4 (Color, 1 / 30s, 30IRE) 0Lux / F1.4 (IR) S/N Ratio More than 50dB IR Distance Up to 50m (164ft) IR On / Off Control Auto / Manual IR LED 3 Lens Lens Type Motorized / Auto Iris (HALL) Mounting Type Board-in Focal Length 2.7 ~ 12mm Motorized Max. Aperture F1.4 Angle of View H: 110 ˚ ~ 36 ˚ V: 57 ˚ ~ 20 ˚ Optical Zoom 4x Sharpness Adjustment Motorized Close Focus Distance 0.5m (1.64ft) Pan / Tilt / Rotation Pan / Tilt / Rotation Left to right: 0° ~ 355°; 0°~65° from top to bottom; rotation: 0 ° ~ 355° 3-Axis (3 Axis) direction adjustment and can be adjusted as desired |  |  |  |
|  | Smart Functions IVS (Optional) Tripwire, Intrusion, Abandoned / Missing Object Advanced Intelligent Functions Face Detection, People Counting, Heat Map Video Compression H.265 + / H.265/H.264 + / H.264 Video Streaming Capacity Triple Streaming Capacity Resolution 1080P (1920 × 1080) /1.3M (1280×960) /720P (1280×720) / D1 (704 × 576 / 704×480) / VGA (640×480) / CIF (352 × 288 / 342×240) ) Frame Rate Main Stream: 1080P (1 ~ 50 / 60fps) SubStream 1: D1 (1~50/60fps) SubStream 2: 1080P (1~50/60fps) Bitrate Control CBR / VBR Bitrate H.264: 24 ~ 10240Kbps H.265: 14 ~ 6144Kbps Day / Night Auto (ICR) / Color / B / W BLC Mode BLC / HLC / WDR (140dB) / SSA White Balance Auto / Natural / Street Light / On / Manual Gain Control Auto / Manual Noise Reduction 3D DNR Motion Detection Off / On (4 Zone, Rectangle) Related Area (4 Zones) On / Off Electronic Image Stabilization (EIS) Available Smart IR Available Ulta Defog (Anti Fog) Available |  |  |  |
|  | Digital Zoom 16x Flip 0°/90°/180°/270° Mirror Off / On Privacy Masking Off / On (4 Areas, Rectangle)" |  |  |  |
| 6 | Photo Booth 1. The proposed Photo Booth must work with the face recognition system. 2. Photo Booth will take automatic biometric photography with face detection system 3. The photo taking system should not take pictures without seeing the face of the person sitting in it. And this whichever language he/she chooses should be spoken in Turkish or English. 4. There should be voice and written guidance in Turkish and English in the photo booth. 5. It should be able to print a photo in 1-2-3-4 different sizes at the same time in the Photo Booth, and this print The dimensions should be changed instantly from the control panel as desired. 6. Normally on the same photo paper when taking biometric photos at the same time in the Photo Booth passport photos can also be printed. 7. Photo Booth should take 2 or 4 different poses, if the user doesn't like it, 2 or 4 different poses for the last time should be able to pose. The user should be able to select the photo on the screen from these. 8. Photo Booth should be able to be set from the control panel, either free or paid. The firm has this will undertake in writing during the inspection. 9. The interior lighting of the photo booth should be LED. It should not be a flash system. |  |  |  |
|  | 10. Taking pictures with a fullhd camera in the photo booth and with a different camera when requested should be done. 11. The photo booth screen must be a touch screen. The user can operate on this screen. should do. 12. The photos taken in the photo booth should be able to be followed online. System automatic It should update itself for 5-6 minutes and use this system online from computer or mobile. can be followed instantly. The company will undertake this issue in writing during the inspection. 13. The photo booth can work with coupons in the online system. Each coupon only once is not used. The company will undertake this issue in writing during the inspection. |  |  |  |
| 7 | Fish Eye Camera 1) 5.0 MP resolution * 1.05 mm m12 lens * 120 db WDR * 8 meters Night Vision distance 2) H.265, H.265+, H.264+, H.264, MJPEG video compression ONVIF(PROFILE S, PROFILE G) PSIA, CGI, ISAPI 3) It should support 128GB memory card. 4) Shutter 1/100,000 5) -30°C / +60°C operating temperature 12 V DC PoE (802.3af) intelligent analysis (Line crossing violation, Intrusion detection, 6) Dynamic analysis, Motion detection) 7) BLC, ROI, Defog, AGC 8) 3D DNR |  |  |  |
| 8 | Car Indoor camera At least 5 MP resolution - color recording- At least 2 gb memory |  |  |  |